AIM: Simulate Discrete time Sequences

To plot unit impulse signal using Python.

In signal processing, a unit impulse signal, also known as a Dirac delta function or impulse  function, is a mathematical function that has the value of 1 at time zero and zero elsewhere.  Generating a unit impulse signal in Python can be done using the NumPy library. Steps:

∙ First, we import the necessary libraries: numpy for numerical computations and  matplotlib.pyplot for plotting.

∙ Next, we define a function called unit_impulse that takes two arguments: length (the  length of the signal) and position (the position of the impulse within the signal). ∙ Inside the unit_impulse function, we create an array of zeros with the specified length  using np.zeros(length).

∙ We set the value at the specified position to 1, representing the impulse. ∙ We define the parameters , , and to specify the x-axis range and step size.

start

stop

step

∙ We use np.arange to generate the x-axis values. The np.arange function creates an array  of numbers from start to stop (inclusive) with a step size of step.

∙ We calculate the length of the impulse signal using len(x), which represents the number  of x-axis values.

∙ We modify the unit_impulse function call to calculate the position of the impulse based  on the x-axis range. Since we want the impulse at n=0, we set the position to  abs(start)//step, which calculates the index of 0 in the x-axis array.

∙ abs(start): The abs() function returns the absolute value of the start variable. This is  done to ensure that the result is always positive, regardless of whether start is positive  or negative.

∙ abs(start)//step: The // operator performs integer division, which discards the decimal  part of the division result and returns the integer quotient. In this case, abs(start)//step  calculates the number of steps from the start value to reach the position of 0 in the x axis array.

∙ The unit_impulse function returns the generated signal.

∙ By using abs(start)//step as the position argument in the unit_impulse function call, we  ensure that the impulse is correctly positioned at n=0 within the generated unit impulse  signal.

∙ For example, if start = -10 and step = 1, the expression abs(start)//step evaluates to  (10)//(1), which equals 10. This means that the impulse will be placed at index 10 in  the signal, aligning it with n=0.

∙ Using abs(start)//step allows the code to handle both positive and negative start values  correctly, ensuring that the unit impulse is positioned accurately regardless of the  specified x-axis range and step size.

∙ We then define the desired length and position for the unit impulse signal. ∙ We generate the unit impulse signal by calling the unit_impulse function with the  specified parameters.

∙ Finally, we plot the unit impulse signal using plt.stem to create a stem plot, and then  display the plot using plt.show().

∙ When you run this code, it will generate a stem plot showing the unit impulse signal  with a spike at the specified position (in this case, position 10).

∙ plt.grid(True) in the code helps to improve the visual representation of the plot by  adding gridlines, making it easier to analyze.

∙ Note: To run this code, you'll need to have the NumPy and Matplotlib libraries installed.  You can install them using pip install numpy matplotlib.

Program:

import numpy as np

import matplotlib.pyplot as plt

def unit_impulse(length, position):

signal = np.zeros(length)

signal[position] = 1

return signal

# Parameters

start = -10 # Start value of the x-axis range

stop = 10 # Stop value of the x-axis range

step = 1 # Step size

# Generate x-axis values

x = np.arange(start, stop+step, step)

# Generate unit impulse signal

impulse_signal = unit_impulse(len(x), abs(start)//step)

# Plot the signal

plt.stem(x, impulse_signal)

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.title('Unit Impulse Signal')

plt.grid(True)

plt.show()

Output:

AIM

Simulate impulse train

Theory:

An impulse train, also known as a Dirac comb, is a periodic sequence of impulses. Each

impulse has an amplitude of 1 and occurs at regular intervals. The impulse train is often used

in signal processing to model periodic phenomena or to analyze the frequency response of

systems.

The impulse train is defined as follows:

impulse_train(n, period) = 1, if n % period == 0

= 0, otherwise

where n is the sample index and period is the period of the impulse train.

Flowchart:

The flowchart for the program will consist of the following steps:

Define the parameters for the impulse train: signal length and period.

Create an empty array to store the impulse train.

Iterate over each sample index.

Check if the sample index is a multiple of the period. If yes, assign the value 1 to the impulse

train array at that index; otherwise, assign the value 0.

Plot and display the impulse train.

Save the impulse train array (optional).

Now, let's see the Python program that simulates an impulse train:

Program

import numpy as np

import matplotlib.pyplot as plt

def simulate_impulse_train(signal_length, period):

impulse_train = np.zeros(signal_length)

for n in range(signal_length):

if n % period == 0:

impulse_train[n] = 1

return impulse_train

# Define the parameters for the impulse train

signal_length = 100 # Length of the impulse train

period = 10 # Period of the impulse train

# Simulate the impulse train

impulse_train = simulate_impulse_train(signal_length, period)

# Plot and display the impulse train

plt.stem(impulse_train)

plt.title('Impulse Train')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.show()

# Save the impulse train array (optional)

# np.savetxt('impulse_train.txt', impulse_train, delimiter=',')

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_impulse_train function takes the signal length and period as input and returns

an array representing the impulse train.

In the main part of the program, we define the parameters for the impulse train: signal_length

(the length of the impulse train) and period (the period of the impulse train).

We simulate the impulse train by calling the simulate_impulse_train function with the

specified parameters.

We plot and display the impulse train using matplotlib.pyplot.stem.

The impulse train array can be optionally saved to a file using numpy.savetxt.

You can modify the signal_length and period variables to change the length and period of the

impulse train, respectively.

Uncomment the np.savetxt line if you want to save the impulse train array to a text file.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

A stem plot will be displayed showing the impulse train.

The impulse train array will be saved to a text file (optional).

Feel free to experiment with different parameters for the impulse train and observe the

output.

AIM: Write a python program to Simulate continuous and discrete unit step signal,

Theory:

A unit step signal, also known as a Heaviside step function, is a signal that is 0 for negative

time or indices and 1 for non-negative time or indices. It is commonly used to represent a

sudden change or transition in a system.

The continuous unit step function is defined as:

u(t) = 0, t < 0

= 1, t >= 0

The discrete unit step function is defined as:

u[n] = 0, n < 0

= 1, n >= 0

Flowchart:

The flowchart for the program will consist of the following steps:

Define the time range (for the continuous signal) or the number of samples (for the discrete

signal).

Create an empty array to store the unit step signal.

Iterate over each time or sample index.

Set the value of the unit step signal to 0 for negative time or indices, and 1 for non-negative

time or indices.

Plot and display the unit step signal.

Save the unit step signal array (optional).

Now, let's see the Python program that simulates continuous and discrete unit step signals:

Program:

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_unit_step(time):

unit_step = np.zeros_like(time)

unit_step[time >= 0] = 1

return unit_step

def simulate_discrete_unit_step(num_samples):

unit_step = np.zeros(num_samples)

unit_step[num_samples // 2:] = 1

return unit_step

# Define the time range for the continuous unit step signal

time = np.linspace(-5, 5, 1000) # Time range from -5 to 5

# Simulate the continuous unit step signal

continuous_unit_step = simulate_continuous_unit_step(time)

# Define the number of samples for the discrete unit step signal

num_samples = 20 # Number of samples

# Simulate the discrete unit step signal

discrete_unit_step = simulate_discrete_unit_step(num_samples)

# Plot and display the continuous and discrete unit step signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_unit_step)

plt.title('Continuous Unit Step Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_unit_step)

plt.title('Discrete Unit Step Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the unit step signal arrays (optional)

# np.savetxt('continuous_unit_step.txt', continuous_unit_step, delimiter=',')

# np.savetxt('discrete_unit_step.txt', discrete_unit_step, delimiter=',')

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_continuous_unit_step function takes a time array as input and returns an array

representing the continuous unit step signal. It creates an array of zeros with the same shape

as the input time array and sets the values to 1 for non-negative time values.

The simulate_discrete_unit_step function takes the number of samples as input and returns an

array representing the discrete unit step signal. It creates an array of zeros with the specified

number of samples and sets the values to 1 starting from the middle index.

In the main part of the program, we define the time range for the continuous unit step signal

using numpy.linspace.

We simulate the continuous unit step signal by calling the simulate_continuous_unit_step

function with the time array.

We define the number of samples for the discrete unit step signal.

We simulate the discrete unit step signal by calling the simulate_discrete_unit_step function

with the number of samples.

We plot and display the continuous and discrete unit step signals using matplotlib.pyplot.plot

and matplotlib.pyplot.stem, respectively.

The unit step signal arrays can be optionally saved to text files using numpy.savetxt.

You can modify the time range for the continuous unit step signal and the num_samples for

the discrete unit step signal according to your requirements.

Uncomment the np.savetxt lines if you want to save the unit step signal arrays to text files.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

Two plots will be displayed: the continuous unit step signal and the discrete unit step signal.

The unit step signal arrays will be saved to text files (optional).

Feel free to experiment with different time ranges and numbers of samples to generate unit

step signals of various lengths.

AIM: Write a python program to Simulate continuous and discrete ramp signal,

Theory:

A ramp signal is a signal that varies linearly with time. It starts at an initial value and increases

or decreases at a constant rate. Ramp signals are commonly used in various applications, such

as signal processing, control systems, and digital communications.

The continuous ramp signal is defined as:

ramp(t) = a * t, for t >= 0

= 0, for t < 0

where a is the slope of the ramp.

The discrete ramp signal is defined as:

ramp[n] = a * n, for n >= 0

= 0, for n < 0

where a is the slope of the ramp.

Flowchart:

The flowchart for the program will consist of the following steps:

Define the time range (for the continuous signal) or the number of samples (for the discrete

signal).

Create an empty array to store the ramp signal.

Iterate over each time or sample index.

Set the value of the ramp signal to a * t for continuous or a * n for discrete, where a is the

slope and t or n is the time or sample index.

Plot and display the ramp signal.

Save the ramp signal array (optional).

Now, let's see the Python program that simulates continuous and discrete ramp signals:

Program

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_ramp(time, slope):

ramp = np.zeros_like(time)

ramp[time >= 0] = slope * time[time >= 0]

return ramp

def simulate_discrete_ramp(num_samples, slope):

ramp = np.zeros(num_samples)

ramp[num_samples // 2:] = slope * np.arange(num_samples // 2, num_samples)

return ramp

# Define the time range for the continuous ramp signal

time = np.linspace(-5, 5, 1000) # Time range from -5 to 5

# Define the number of samples and slope for the discrete ramp signal

num_samples = 20 # Number of samples

slope = 2 # Slope of the ramp

# Simulate the continuous ramp signal

continuous_ramp = simulate_continuous_ramp(time, slope)

# Simulate the discrete ramp signal

discrete_ramp = simulate_discrete_ramp(num_samples, slope)

# Plot and display the continuous and discrete ramp signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_ramp)

plt.title('Continuous Ramp Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_ramp)

plt.title('Discrete Ramp Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the ramp signal arrays (optional)

# np.savetxt('continuous_ramp.txt', continuous_ramp, delimiter=',')

# np.savetxt('discrete_ramp.txt', discrete_ramp, delimiter=',')

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_continuous_ramp function takes a time array and a slope as input and returns an

array representing the continuous ramp signal. It creates an array of zeros with the same shape

as the input time array and sets the values to slope * time for non-negative time values.

The simulate_discrete_ramp function takes the number of samples and a slope as input and

returns an array representing the discrete ramp signal. It creates an array of zeros with the

specified number of samples and sets the values to slope * n starting from the middle index.

In the main part of the program, we define the time range for the continuous ramp signal using

numpy.linspace.

We define the number of samples and slope for the discrete ramp signal.

We simulate the continuous ramp signal by calling the simulate_continuous_ramp function

with the time array and slope.

We simulate the discrete ramp signal by calling the simulate_discrete_ramp function with the

number of samples and slope.

We plot and display the continuous and discrete ramp signals using matplotlib.pyplot.plot and

matplotlib.pyplot.stem, respectively.

The ramp signal arrays can be optionally saved to text files using numpy.savetxt.

You can modify the time range for the continuous ramp signal, the num_samples for the

discrete ramp signal, and the slope of the ramps according to your requirements.

Uncomment the np.savetxt lines if you want to save the ramp signal arrays to text files.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

Two plots will be displayed: the continuous ramp signal and the discrete ramp signal.

The ramp signal arrays will be saved to text files (optional).

Feel free to experiment with different time ranges, numbers of samples, and slopes to

generate ramp signals of various lengths and slopes.

AIM: Write a python program to Simulate continuous and discrete exponential signal,

Theory:

An exponential signal is a signal whose amplitude changes exponentially with time or index.

Exponential signals can exhibit either growth or decay behavior, depending on the sign of the

exponential term.

The continuous exponential signal is defined as:

x(t) = A * exp(b * t)

where A is the initial amplitude, b is the exponential coefficient, and t is the time.

The discrete exponential signal is defined as:

x[n] = A * exp(b * n)

where A is the initial amplitude, b is the exponential coefficient, and n is the sample index.

Flowchart:

The flowchart for the program will consist of the following steps:

Define the time range (for the continuous signal) or the number of samples (for the discrete

signal).

Create an empty array to store the exponential signal.

Iterate over each time or sample index.

Set the value of the exponential signal to A * exp(b * t) for continuous or A * exp(b * n) for

discrete, where A is the initial amplitude, b is the exponential coefficient, and t or n is the

time or sample index.

Plot and display the exponential signal.

Save the exponential signal array (optional).

Now, let's see the Python program that simulates continuous and discrete exponential signals:

Program

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_exponential(time, amplitude, coefficient):

exponential_signal = amplitude * np.exp(coefficient * time)

return exponential_signal

def simulate_discrete_exponential(num_samples, amplitude, coefficient):

exponential_signal = amplitude * np.exp(coefficient * np.arange(num_samples))

return exponential_signal

# Define the time range for the continuous exponential signal

time = np.linspace(0, 5, 1000) # Time range from 0 to 5

# Define the number of samples, initial amplitude, and coefficient for the discrete exponential

signal

num_samples = 20 # Number of samples

amplitude = 2 # Initial amplitude

coefficient = -0.5 # Exponential coefficient

# Simulate the continuous exponential signal

continuous_exponential = simulate_continuous_exponential(time, amplitude, coefficient)

# Simulate the discrete exponential signal

discrete_exponential = simulate_discrete_exponential(num_samples, amplitude, coefficient)

# Plot and display the continuous and discrete exponential signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_exponential)

plt.title('Continuous Exponential Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_exponential)

plt.title('Discrete Exponential Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the exponential signal arrays (optional)

# np.savetxt('continuous_exponential.txt', continuous_exponential, delimiter=',')

# np.savetxt('discrete_exponential.txt', discrete_exponential, delimiter=',')

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_continuous_exponential function takes a time array, initial amplitude, and

exponential coefficient as input and returns an array representing the continuous exponential

signal. It calculates the exponential signal using numpy.exp with the specified amplitude and

coefficient.

The simulate_discrete_exponential function takes the number of samples, initial amplitude, and

exponential coefficient as input and returns an array representing the discrete exponential

signal. It calculates the exponential signal using numpy.exp with the specified amplitude and

coefficient, applied element-wise to the range of sample indices.

In the main part of the program, we define the time range for the continuous exponential signal

using numpy.linspace.

We define the number of samples, initial amplitude, and exponential coefficient for the discrete

exponential signal.

We simulate the continuous exponential signal by calling the simulate_continuous_exponential

function with the time array, amplitude, and coefficient.

We simulate the discrete exponential signal by calling the simulate_discrete_exponential

function with the number of samples, amplitude, and coefficient.

We plot and display the continuous and discrete exponential signals using

matplotlib.pyplot.plot and matplotlib.pyplot.stem, respectively.

The exponential signal arrays can be optionally saved to text files using numpy.savetxt.

You can modify the time range for the continuous exponential signal, the number of samples,

initial amplitude, and exponential coefficient for the discrete exponential signal according to

your requirements.

Uncomment the np.savetxt lines if you want to save the exponential signal arrays to text files.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

Two plots will be displayed: the continuous exponential signal and the discrete exponential

signal.

The exponential signal arrays will be saved to text files (optional).

Feel free to experiment with different time ranges, numbers of samples, initial amplitudes,

and exponential coefficients to generate exponential signals of various shapes and

characteristics.

AIM: Write a python program to Simulate continuous and discrete parabolic signal

Theory:

A parabolic signal is a signal whose amplitude changes quadratically with time or index. It

follows a parabolic curve and is commonly used to model various phenomena, such as

projectile motion, signal processing, and data fitting.

The continuous parabolic signal is defined as:

parabolic(t) = a * t^2 + b * t + c

where a, b, and c are coefficients and t is the time.

The discrete parabolic signal is defined as:

parabolic[n] = a * n^2 + b * n + c

where a, b, and c are coefficients and n is the sample index.

Flowchart:

The flowchart for the program will consist of the following steps:

Define the time range (for the continuous signal) or the number of samples (for the discrete

signal).

Create an empty array to store the parabolic signal.

Iterate over each time or sample index.

Set the value of the parabolic signal to a * t^2 + b * t + c for continuous or a * n^2 + b * n + c

for discrete, where a, b, and c are the coefficients and t or n is the time or sample index.

Plot and display the parabolic signal.

Save the parabolic signal array (optional).

Now, let's see the Python program that simulates continuous and discrete parabolic signals:

Program:

import numpy as np

import matplotlib.pyplot as plt

FACULTY OF ENGINEERING AND TECHNOLOGY

Department of Information and Communication Technology

def simulate_continuous_parabolic(time, coefficients):

parabolic_signal = np.polyval(coefficients, time)

return parabolic_signal

def simulate_discrete_parabolic(num_samples, coefficients):

parabolic_signal = np.polyval(coefficients, np.arange(num_samples))

return parabolic_signal

# Define the time range for the continuous parabolic signal

time = np.linspace(-5, 5, 1000) # Time range from -5 to 5

# Define the number of samples and coefficients for the discrete parabolic signal

num_samples = 20 # Number of samples

coefficients = [1, 2, 1] # Coefficients of the parabolic signal

# Simulate the continuous parabolic signal

continuous_parabolic = simulate_continuous_parabolic(time, coefficients)

# Simulate the discrete parabolic signal

discrete_parabolic = simulate_discrete_parabolic(num_samples, coefficients)

# Plot and display the continuous and discrete parabolic signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_parabolic)

plt.title('Continuous Parabolic Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_parabolic)

plt.title('Discrete Parabolic Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the parabolic signal arrays (optional)

# np.savetxt('continuous_parabolic.txt', continuous_parabolic, delimiter=',')

# np.savetxt('discrete_parabolic.txt', discrete_parabolic, delimiter=',')

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_continuous_parabolic function takes a time array and a list of coefficients as

input and returns an array representing the continuous parabolic signal. It calculates the

parabolic signal using numpy.polyval with the specified coefficients and time array.

FACULTY OF ENGINEERING AND TECHNOLOGY

Department of Information and Communication Technology

The simulate_discrete_parabolic function takes the number of samples and a list of coefficients

as input and returns an array representing the discrete parabolic signal. It calculates the

parabolic signal using numpy.polyval with the specified coefficients and a range of sample

indices.

In the main part of the program, we define the time range for the continuous parabolic signal

using numpy.linspace.

We define the number of samples and coefficients for the discrete parabolic signal.

We simulate the continuous parabolic signal by calling the simulate_continuous_parabolic

function with the time array and coefficients.

We simulate the discrete parabolic signal by calling the simulate_discrete_parabolic function

with the number of samples and coefficients.

We plot and display the continuous and discrete parabolic signals using matplotlib.pyplot.plot

and matplotlib.pyplot.stem, respectively.

The parabolic signal arrays can be optionally saved to text files using numpy.savetxt.

You can modify the time range for the continuous parabolic signal, the number of samples, and

the coefficients for the discrete parabolic signal according to your requirements.

Uncomment the np.savetxt lines if you want to save the parabolic signal arrays to text files.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

Two plots will be displayed: the continuous parabolic signal and the discrete parabolic signal.

The parabolic signal arrays will be saved to text files (optional).

Feel free to experiment with different time ranges, numbers of samples, and coefficients to

generate parabolic signals of various shapes and characteristics.

AIM: Write a python program to Simulate continuous and discrete sine wave signal

Theory:

A sine wave signal is a continuous-time or discrete-time signal that follows a periodic

oscillation described by the sine function. It is widely used in various applications, such as

signal processing, communications, and audio synthesis.

The continuous sine wave signal is defined as:

sine(t) = A * sin(2 * pi * f * t + phi)

where A is the amplitude, f is the frequency, t is the time, and phi is the phase.

The discrete sine wave signal is defined as:

sine[n] = A * sin(2 * pi * f * n / Fs + phi)

where A is the amplitude, f is the frequency, n is the sample index, Fs is the sampling

frequency, and phi is the phase.

Flowchart:

The flowchart for the program will consist of the following steps:

Define the time range (for the continuous signal) or the number of samples and the sampling

frequency (for the discrete signal).

Create an empty array to store the sine wave signal.

Iterate over each time or sample index.

Set the value of the sine wave signal to A * sin(2 * pi * f * t + phi) for continuous or A *

sin(2 * pi * f * n / Fs + phi) for discrete, where A, f, t, n, Fs, and phi are the corresponding

parameters.

Plot and display the sine wave signal.

Save the sine wave signal array (optional).

Now, let's see the Python program that simulates continuous and discrete sine wave signals:

Program:

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_sine_wave(time, amplitude, frequency, phase):

sine_wave = amplitude * np.sin(2 * np.pi * frequency * time + phase)

return sine_wave

def simulate_discrete_sine_wave(num_samples, sampling_frequency, amplitude, frequency,

phase):

time = np.arange(num_samples) / sampling_frequency

sine_wave = amplitude * np.sin(2 * np.pi * frequency * time + phase)

return sine_wave

# Define the time range for the continuous sine wave signal

time = np.linspace(0, 1, 1000) # Time range from 0 to 1 second

# Define the number of samples, sampling frequency, and parameters for the discrete sine

wave signal

num_samples = 100 # Number of samples

sampling_frequency = 10 # Sampling frequency in Hz

amplitude = 1 # Amplitude of the sine wave

frequency = 2 # Frequency of the sine wave in Hz

phase = 0 # Phase angle of the sine wave in radians

# Simulate the continuous sine wave signal

continuous_sine_wave = simulate_continuous_sine_wave(time, amplitude, frequency, phase)

# Simulate the discrete sine wave signal

discrete_sine_wave = simulate_discrete_sine_wave(num_samples, sampling_frequency,

amplitude, frequency, phase)

# Plot and display the continuous and discrete sine wave signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_sine_wave)

plt.title('Continuous Sine Wave Signal')

plt.xlabel('Time (s)')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_sine_wave)

plt.title('Discrete Sine Wave Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the sine wave signal arrays (optional)

# np.savetxt('continuous_sine_wave.txt', continuous_sine_wave, delimiter=',')

# np.savetxt('discrete_sine_wave.txt', discrete_sine_wave, delimiter=',')

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_continuous_sine_wave function takes a time array, amplitude, frequency, and

phase as input and returns an array representing the continuous sine wave signal. It calculates

the sine wave signal using numpy.sin with the appropriate formula.

The simulate_discrete_sine_wave function takes the number of samples, sampling frequency,

amplitude, frequency, and phase as input and returns an array representing the discrete sine

wave signal. It first calculates the time array using the number of samples and sampling

frequency and then calculates the sine wave signal using numpy.sin with the appropriate

formula.

In the main part of the program, we define the time range for the continuous sine wave signal

using numpy.linspace.

We define the number of samples, sampling frequency, amplitude, frequency, and phase for

the discrete sine wave signal.

We simulate the continuous sine wave signal by calling the simulate_continuous_sine_wave

function with the time array and parameters.

We simulate the discrete sine wave signal by calling the simulate_discrete_sine_wave function

with the number of samples, sampling frequency, amplitude, frequency, and phase.

We plot and display the continuous and discrete sine wave signals using matplotlib.pyplot.plot

and matplotlib.pyplot.stem, respectively.

The sine wave signal arrays can be optionally saved to text files using numpy.savetxt.

You can modify the time range for the continuous sine wave signal, the number of samples,

sampling frequency, amplitude, frequency, and phase for the discrete sine wave signal

according to your requirements.

Uncomment the np.savetxt lines if you want to save the sine wave signal arrays to text files.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

Two plots will be displayed: the continuous sine wave signal and the discrete sine wave

signal.

The sine wave signal arrays will be saved to text files (optional).

Feel free to experiment with different time ranges, numbers of samples, sampling

AIM: Write a python program to simulate y(t)=Delta(t)+delta(t-1)+3*delta(t+5).

Theory:

The impulse function, denoted as Delta(t), is a function that is infinite at t=0 and zero

elsewhere. It is often used to model instantaneous events.

The shifted impulse function, denoted as delta(t), is similar to the impulse function but is shifted

by a certain amount. It is non-zero only at the shifted time.

The function y(t) = Delta(t) + delta(t-1) + 3*delta(t+5) is a combination of three impulse

functions. It has an impulse at t=0, t=1, and t=-5, with different magnitudes.

Flowchart:

The flowchart for the program will consist of the following steps:

Define the time range.

Create an empty array to store the function values.

Iterate over each time index.

Calculate the value of y(t) at each time index using the given function formula.

Plot and display the function values.

Now, let's see the Python program that simulates y(t) = Delta(t) + delta(t-1) +

3*delta(t+5):

import numpy as np

import matplotlib.pyplot as plt

def simulate_function(time):

y = np.zeros_like(time)

y[time == 0] = 1

y[time == 1] += 1

y[time == -5] += 3

return y

# Define the time range

time = np.arange(-10, 11)

# Simulate the function

function_values = simulate_function(time)

# Plot and display the function

plt.stem(time, function_values)

plt.title('Function y(t) = Delta(t) + delta(t-1) + 3*delta(t+5)')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.ylim([-0.5, 4.5])

plt.grid(True)

plt.show()

Explanation:

We import the required modules, including numpy for array operations and matplotlib.pyplot

for plotting.

The simulate_function function takes a time array as input and returns an array representing

the function values. It creates an array of zeros with the same shape as the input time array and

sets the values according to the function formula.

In the main part of the program, we define the time range using numpy.arange.

We simulate the function by calling the simulate_function function with the time array.

We plot and display the function values using matplotlib.pyplot.stem.

We set the title, x-axis label, y-axis label, y-axis limits, and enable the grid.

Finally, we show the plot using matplotlib.pyplot.show.

You can modify the time range according to your requirements.

Make sure you have numpy and matplotlib installed (pip install numpy matplotlib) before

running this program.

Expected Output:

A stem plot will be displayed showing the function y(t) = Delta(t) + delta(t-1) + 3*delta(t+5).

The function will have impulses at t=0, t=1, and t=-5, with magnitudes of 1, 1, and 3

respectively, and zero elsewhere.